Sowerby Wood Industrial Estate

Barrow-in-Furness

Cumbria,

LA14 4RF

Date: 15/ 07/ 2026

Ref: P8443 SPIDA Datalogging Rev.A

Dear Duncan,

Please find below an outline of our proposal for The SPIDA Datalogging System.

Proposed Data Logging System

System Overview

A dedicated industrial data logging system will be provided to record, display and archive operational data from the SPIDA platform. The system will communicate directly with the existing Beckhoff CX5240 PLC using the PLC's native OPC UA Server, providing a secure and reliable interface without requiring any modifications to the control system.

The data logging computer will be installed within a lockable, weatherproof industrial enclosure located at the base of the chimney. Communications between the PLC and the logging system will be established via a fibre optic Ethernet link, installed by others, utilising Ethernet-to-fibre media converters at each end. The fibre connection provides electrical isolation, excellent noise immunity and a robust communications path suitable for the installation environment.

Data Collection and Historian

The system will utilise Inductive Automation Ignition as the data acquisition platform, with the Tag Historian module providing continuous collection and long-term storage of process information. Selected PLC variables will be acquired over OPC UA and recorded in a time-stamped historical database for subsequent review and analysis.

The historian will record all key operating parameters of the lifting platform, including:

Individual hanger loads

Individual hanger positions

Hydraulic pressures

Lock pin status

Horizontal support status

Overall platform load

Additional applied loads

System alarms and events

Historical records will allow operational trends to be reviewed, provide evidence of platform loading throughout lifting operations, and assist with maintenance, fault diagnosis and post-operation analysis.

Visualisation

A local operator interface will be provided using Ignition Perspective, allowing operators and maintenance personnel to view both live and historical information from the logging system.

The display will provide real-time visualisation of all logged process values, together with historical trends, equipment status and alarm information through an intuitive browser-based interface. The architecture also provides flexibility for future expansion should additional displays or remote viewing be required.

Data Export

Historical data will be available for export directly from the historian database. Authorised users will be able to extract recorded data to a USB memory device in standard formats such as CSV, enabling straightforward transfer of operational records for reporting, engineering analysis or long-term archiving.

Testing

The Costs Include for Testing of the System Off-Site against a PLC Configured with the Identical Software to the SPIDA.

Installation

The Costs do Not Include Site Installation, Cabling to The SPIDA, Supply of Suitable Power to the Enclosure.

Summary

The proposed solution provides a robust, industrial-grade data logging system that combines Beckhoff OPC UA communications with Ignition's historian and visualisation capabilities. Installed within a weatherproof enclosure at the base of the chimney and connected via a Summary

The proposed solution provides a robust, industrial-grade data logging system that combines Beckhoff OPC UA communications with Ignition's historian and visualisation capabilities. Installed within a weatherproof enclosure at the base of the chimney and connected via a dedicated fibre optic Ethernet link, the system will deliver reliable acquisition, long-term storage and real-time visualisation of critical SPIDA operating data, together with comprehensive alarm logging and simple export of historical records for engineering and maintenance purposes.

Indicative Timeline

We would expect the system to be completed and delivered within 8-12 weeks from receipt of order.

The proposed start date would be the 01th August 2026.

Cost

| Item | Description | Cost | Qty | Total |
| --- | --- | --- | --- | --- |
| 1 | Supply Datalogging System in Industrial Enclosure (Ex.Works) | £ 54,750.00 | 1 | £ 54,750.00 |
|  |  | Total |  | £ 54,750.00 |

All Cost Exclude VAT at Standard Rate.

All payments to UK Bank Account in GBP.

Payment Terms

25% with Order

50% on Build Completion

25% on FAT Test, before Delivery.

30 Days Nett

Validity

Costs are Valid for 30 Days from Date of Proposal.

Kind regards,

M.Bird

Malcolm Bird.

Managing Director

Swift Automation & Control Systems Ltd.

Mob: +44 (0) 7976 259109